# PFAS Weekly Update to LHD Advisory Committee January 7, 2022

Site Highlights

- Eagle Ottawa Farms (new site Oceana and Newaygo Counties)

- New PFAS site where waste materials from the former Eagle Ottawa Leather Company was sent to a Combined Animal Feeding Operation (CAFO) for use as compost.  The compost material was spread over two nearby farms.  As part of a Baseline Environmental Assessment (BEA) for the two farms, PFAS was detected in soil and groundwater samples.  PFOA and PFOS were detected in groundwater samples at both farms at concentrations exceeding Part 201 drinking water criteria.  PFOA was detected at a maximum concentration of 1,800 ppt and PFOS was detected at maximum concentration of 240 ppt.  Project team will be working on identifying nearby residential wells in the area for targeted sampling.